# System Controller Firmware Release Notes

# Release Notes:

V1p94:        						Date: 02-July-2026

- Modified the XVC relay logic to prevent unintended momentary relay closure in slave units when the master unit shuts down due to an error condition.

- After a *RST command or power cycle, the selected IEC413 class is now retained and automatically restored. This allows customers to continue IEC413 testing without output voltage dropouts or the need to reconfigure the setup after restart.

- Fixed an issue in XVC mode where the default waveform functions (Square and Clip) could not be selected or programmed.

- Fixed an issue where waveforms with numeric names were not properly restored after a power cycle or *RST.

- Updated the implementation to prevent automatic range switching duringIEC411 operation.

- Added the SCPI command to switch between the standard and EHF loop in the Source mode based on the customer application.

- Added support for new Option F47.

V1p83:        						Date: 7-July-2025

- Logic is added to firmware to take Low range DC offset calibration value in high range if it did not calibrate in high range.

- GPIB driver is updated to solve the communication dropout issues.

- Updated SCPI command to support up to 12 units.

- Added the SCPI command to switch between the standard and EHF loop in the Source mode based on the customer application.

V1p80:        						Date: 28-Mar-2025

- Added new feature to the change operating mode (Power supply, grid sim and Eload) without power cycling the system.

- Updated the number of units to be paralleled to 12.

V1p78:        						Date: 07-Jan-2025

- To adjust Low range DC offset, added new calibration in the latest version. After updating firmware from 1.66 version to 1.78, need to update low range calibration as following

Cal:pass “5000”

Cal:volt:Hrange:offset?

Response: val_offset

Cal:volt:lrange:offset <val_offset>

Cal:volt:lrange:hfoff <val_offset>

- Added the coefficients to calibrate the DC offset in the 666 volts model – To reduce the excitation current of the output transformer.

- OVP limits are automatically update depends on voltage range.

- Serial communication issue while power cycling unit is fixed.

- Ethernet and GPIB communication failure upon activating SRQ bit is solved.

V1p66:								Date: 13-March-2024

- Improve the Phase shift accuracy in EL mode.

V1p65:								Date: 11-Oct-2023

- XVC option maximum current calculation issue is solved.

- PONS current setting issue is solved.

- PONS over voltage protection max query issue is solved.

- Module fault status Asci format is updated.

### V1p63:								Date: 25-July-2023

- Disabled Frequency Control in ELOAD even with FC option.

- CC mode to CV mode transition issues is solved.

### V1p61:								Date: 08-June-2023

- Initial Release