# System Controller Firmware Release Notes

# Release Notes:

V1p80:        						Date: 28-Mar-2025

- Added new feature to the change operating mode (Power supply, grid sim and Eload) without power cycling the system.

- Updated the number of units to be paralleled to 12.

V1p78:        						Date: 07-Jan-2025

- To adjust Low range DC offset, added new calibration in the latest version. After updating firmware from 1.66 version to 1.78, need to update low range calibration as following

Cal:pass “5000”

Cal:volt:Hrange:offset?

Response: val_offset

Cal:volt:lrange:offset <val_offset>

Cal:volt:lrange:hfoff <val_offset>

- Added the coefficients to calibrate the DC offset in the 666 volts model – To reduce the excitation current of the output transformer.

- OVP limits are automatically update depends on voltage range.

- Serial communication issue while power cycling unit is fixed.

- Ethernet and GPIB communication failure upon activating SRQ bit is solved.

- In power supply mode frequency support extended up to 1500 Hz.

V1p66:								Date: 13-March-2024

- Improve the Phase shift accuracy in EL mode.

V1p65:								Date: 11-Oct-2023

- XVC option maximum current calculation issue is solved.

- PONS current setting issue is solved.

- PONS over voltage protection max query issue is solved.

- Module fault status Asci format is updated.

### V1p63:								Date: 25-July-2023

- Disabled Frequency Control in ELOAD even with FC option.

- CC mode to CV mode transition issues is solved.

### V1p61:								Date: 08-June-2023

- Initial Release